LNTB

Leading Next-Gen Tomato Business

Hardware, Firmware and AI Design Specification

លក្ខណៈបច្ចេកទេស Hardware Firmware និង AI

PLC & SDLC Documentation Pack

| Document ID | LNTB-HFA-001 | Version | 1.0 |
| --- | --- | --- | --- |
| Status | Baseline for implementation | Date | 2026-07-11 |
| Owner | IoT Lead / AI Lead | Reviewer | Technical Lead / QA Lead |
| Approver | Project Sponsor / Academic Supervisor | Classification | Project Internal |

Document purpose — This controlled document defines the approved baseline for the LNTB pilot. Changes are managed through the Change Register and must preserve traceability to requirements, design, tests and acceptance evidence.

# Document Control

| Version | Date | Author/Owner | Change Summary | Status |
| --- | --- | --- | --- | --- |
| 0.1 | 2026-07-11 | LNTB Project Team | Initial controlled draft | Draft |
| 1.0 | 2026-07-11 | LNTB Project Team | PLC/SDLC baseline for implementation | Baseline |

## Approvals

| Role | Name | Decision | Date | Signature / Evidence |
| --- | --- | --- | --- | --- |
| Project Sponsor / Supervisor | TBD | Pending | TBD | Meeting minutes or signed copy |
| Project Manager | TBD | Pending | TBD | Change/approval record |
| Technical Lead | TBD | Pending | TBD | Architecture review record |
| QA Lead | TBD | Pending | TBD | Quality gate record |

# 1. Design Scope and Safety Principle

This specification defines the pilot electronics, wiring baseline, firmware responsibilities, communications, calibration and AI lifecycle. Final pin assignments and mains-energy measurement installation require review against the exact purchased boards/modules. Electrical work must be performed using appropriately rated, isolated components and a physical means of disconnection.

Fail-safe principle — Loss of software, network or uncertain sensor input must not leave the pump energized. Firmware enforces maximum runtime independently of Laravel, and the installation provides a manual electrical cutoff.

# 2. Revised Bill of Materials

| Item | Qty | Purpose | Planning Cost | Control Note |
| --- | --- | --- | --- | --- |
| ESP32 development board | 1 | Sensor/actuator controller | $7.00 | Original |
| ESP32-CAM + stable power adapter | 1 | Image capture | $12.00 + power | Validate supply current |
| Capacitive soil moisture sensor | 2 | Root-zone moisture | ~$4.00 | Prefer corrosion-resistant type |
| DHT22 / approved equivalent | 1 | Air temperature/humidity | $4.50 | Shield from direct water |
| LDR module + servo | 1 set | Light/roof prototype | $6.00 | Mechanical limit test |
| 2-channel rated relay + pump + fan | 1 set | Actuation | $20.00 | Isolation/fuse/rating review |
| Water-flow meter | 1 | Actual irrigation volume | $8–$20 allowance | New mandatory measurement item |
| Isolated energy meter/module | 1 | Actual pump kWh | $12–$30 allowance | New mandatory measurement item |
| Enclosure, glands, fuse, connectors, PCB/wiring | As needed | Safe field installation | $30.00 + contingency | IP-rated enclosure preferred |
| Mobile app / pilot cloud services | Allowance | Software hosting/test | $20.00 | Track expiry and recurring cost |

# 3. Proposed ESP32 Pin Baseline

| Function | Proposed Pin | Mode | Design Note |
| --- | --- | --- | --- |
| Soil moisture 1 | GPIO34 | ADC input | Input-only; calibrate dry/wet raw range |
| Soil moisture 2 | GPIO35 | ADC input | Input-only; separate root zone |
| LDR | GPIO32 | ADC input | Use voltage divider and validate ADC range |
| DHT22 | GPIO4 | Digital | Pull-up as required; avoid blocking read frequency |
| Flow meter pulse | GPIO33 | Interrupt-capable input | Debounce/filter and persist pulse accumulator |
| Pump relay | GPIO26 | Digital output | Default LOW/OFF; confirm relay active level |
| Fan relay | GPIO27 | Digital output | Default LOW/OFF |
| Servo signal | GPIO25 | PWM | Enforce safe angle limits |
| Energy meter UART RX/TX | GPIO16 / GPIO17 | UART2 | Only for compatible isolated serial meter |

Pin-control note — This is a proposed baseline, not an installation instruction. Confirm ESP32-CAM pin conflicts, boot strapping pins, ADC characteristics, relay active level and purchased meter interfaces before wiring.

# 4. Electrical and Environmental Controls

Use separate stable power capacity for pump/servo/camera as required; share ground only where the approved design requires it.

Protect low-voltage electronics from pump transients using suitable flyback/snubbers, decoupling and physical separation.

Use fuse/over-current protection, strain relief, cable glands and waterproof enclosure appropriate to humid net-house conditions.

Keep mains voltage isolated from ESP32/user-accessible low-voltage circuits. Do not use an unisolated energy module in the field.

Label manual cutoff, relay channels, polarity, device ID and safe service procedure.

Perform dry bench test before connecting water; perform leak and condensation inspection before pilot operation.

# 5. Firmware Modules

| Module | Responsibilities | Key Outputs |
| --- | --- | --- |
| Configuration | Provision Wi-Fi/API/device credentials; load signed/versioned settings; protect secrets | Validated runtime config and fallback settings |
| Sensor manager | Sample/filter moisture, DHT, light, flow and energy; detect implausible values | Timestamped readings + quality flags |
| Safety controller | Enforce pump fail-off, max runtime, minimum rest time, dry-run/invalid-sensor policy | Safe actuator decision and safety event |
| Command manager | Poll/lease, validate, execute and acknowledge commands | Deterministic command transition |
| Irrigation event recorder | Open/close event and accumulate pulses/energy | Duration, m³, kWh, measured flags |
| Offline queue | Persist events/readings with sequence and client_event_id; retry and compact after acknowledgement | At least 24-hour store-and-forward |
| Network client | TLS, timeout, exponential backoff, clock synchronization | Connection and sync status |
| Diagnostics | Watchdog, boot reason, firmware version, error counters and last successful sync | Health payload and serial diagnostics |

# 6. Control State Machine

| State | Entry / Behavior | Exit / Safety |
| --- | --- | --- |
| BOOT_SAFE | Outputs OFF; load config; self-test; recover interrupted event | Move to CONNECTING only with valid safety config |
| CONNECTING | Attempt Wi-Fi/time/API with bounded backoff; sample locally | Move ONLINE when authenticated; OFFLINE_SAFE after timeout |
| ONLINE_IDLE | Sample, upload, poll commands and evaluate approved auto mode | To PUMPING on valid decision; OFFLINE_SAFE on loss |
| PUMPING | Relay ON; meter accumulation; watchdog and max-runtime active | OFF on stop, threshold, fault, max runtime or reboot |
| OFFLINE_SAFE | Local sampling/buffer; approved fallback threshold only | Reconnect to ONLINE; pump remains bounded by local safety |
| FAULT_LOCKOUT | Relay OFF; record critical fault; reject unsafe start | Manual/authorized clear after cause resolved |

# 7. Communications and Synchronization

Every reading/event contains device UID, firmware version, client_event_id, monotonically increasing sequence where available, recorded_at and quality flags.

Use HTTPS with certificate validation. Credentials are provisioned per device and revocable; do not embed a shared secret in public source.

Retry with bounded exponential backoff and jitter. Do not retry unsafe commands blindly.

Backend acknowledgement advances the persisted sync cursor. Duplicate client_event_id is treated as success with the existing server record.

Commands include ID, type, expiry, safety parameters and requested version. Expired, unknown or unsafe commands are rejected and acknowledged with a reason.

# 8. Calibration Plan

| Instrument / Sensor | Method | Acceptance / Evidence |
| --- | --- | --- |
| Soil moisture | Record dry media, saturated/drained media and intermediate reference samples per sensor/soil mix | Stable mapping documented; thresholds validated with farmer/agronomic input |
| Flow meter | Compare pulse-derived volume against a calibrated container across low/normal flow and repeated runs | Target error within ±5%; pulses-per-litre stored with date/device |
| Energy meter | Compare against trusted reference over representative pump loads/durations | Target error within ±5%; wiring/rating evidence retained |
| DHT22 | Compare with a reference thermometer/hygrometer at two conditions | Bias documented; replace if outside agreed tolerance |
| LDR | Map raw value under representative light conditions; define use as trend or calibrated lux | Units and limitations explicit |
| Servo/roof | Set mechanical open/closed angles and verify no binding | Limits recorded; repeated cycle test passes |

# 9. Camera and AI Pipeline

| Stage | Control |
| --- | --- |
| Capture | Consistent distance, angle and lighting guidance; include capture timestamp/device and image checksum. |
| Transfer | Compress to approved size; retry safely; store privately; reject oversized/invalid files. |
| Annotation | Classes: unripe, semi-ripe, ready-to-harvest; label guide and second-review sample; record uncertain cases. |
| Split | Separate training/validation/test by collection session/plant where possible to reduce leakage. |
| Training | Version code, dataset, preprocessing, hyperparameters and random seed; retain model artifact checksum. |
| Validation | Report per-class precision/recall/F1, macro F1, confusion matrix and representative failure examples. |
| Inference | Queue job; return class, confidence, model version and processing status; timeout/retry bounded. |
| Human review | Farmer can inspect image and outcome; low-confidence results do not trigger certainty language. |
| Monitoring | Track confidence distribution, class drift, reviewed disagreements and changed field conditions. |

# 10. Hardware/Firmware/AI Acceptance Tests

| ID | Test | Pass Condition |
| --- | --- | --- |
| HFA-T01 | Power-on/reboot during pump operation | Output initializes OFF and interrupted event/fault is recorded |
| HFA-T02 | Network and backend unavailable | Local safe irrigation and 24-hour buffer operate; no unbounded pump |
| HFA-T03 | Duplicate replay | Backend stores one logical reading/event |
| HFA-T04 | Maximum runtime | Pump stops locally at limit even if no stop command arrives |
| HFA-T05 | Invalid/stuck sensor | No unsafe automatic pump start; quality/fault visible |
| HFA-T06 | Flow/energy calibration | Observed error within approved ±5% target |
| HFA-T07 | Enclosure/field inspection | No exposed conductors/leaks; labels, fuse and cutoff verified |
| HFA-T08 | AI held-out validation | Metrics and limitations reported; macro F1 target assessed without data leakage |
| HFA-T09 | Low-confidence result | UI/notification presents review guidance, not certainty |

# Authoritative References

| ID | Source | Use |
| --- | --- | --- |
| SRC-01 | LNTB Khmer proposal | Goals, scope, BOM and timeline |
| SRC-02 | LNTB requirements | Functional/non-functional baseline |
| SRC-03 | LNTB project structure | Initial architecture, data and APIs |
| SRC-04 | Official Laravel docs | Backend implementation baseline |
| SRC-05 | Official Flutter docs | Mobile architecture/quality baseline |
| SRC-06 | Official Espressif docs | ESP32 hardware/network constraints |