LNTB

Leading Next-Gen Tomato Business

Deployment, Operations, Maintenance and Support Plan

ផែនការដាក់ដំណើរការ ប្រតិបត្តិការ ថែទាំ និងគាំទ្រ

PLC & SDLC Documentation Pack

| Document ID | LNTB-OPS-001 | Version | 1.0 |
| --- | --- | --- | --- |
| Status | Baseline for implementation | Date | 2026-07-11 |
| Owner | DevOps / Operations Lead | Reviewer | Technical Lead / QA Lead |
| Approver | Project Sponsor / Academic Supervisor | Classification | Project Internal |

Document purpose — This controlled document defines the approved baseline for the LNTB pilot. Changes are managed through the Change Register and must preserve traceability to requirements, design, tests and acceptance evidence.

# Document Control

| Version | Date | Author/Owner | Change Summary | Status |
| --- | --- | --- | --- | --- |
| 0.1 | 2026-07-11 | LNTB Project Team | Initial controlled draft | Draft |
| 1.0 | 2026-07-11 | LNTB Project Team | PLC/SDLC baseline for implementation | Baseline |

## Approvals

| Role | Name | Decision | Date | Signature / Evidence |
| --- | --- | --- | --- | --- |
| Project Sponsor / Supervisor | TBD | Pending | TBD | Meeting minutes or signed copy |
| Project Manager | TBD | Pending | TBD | Change/approval record |
| Technical Lead | TBD | Pending | TBD | Architecture review record |
| QA Lead | TBD | Pending | TBD | Quality gate record |

# 1. Release and Environment Strategy

| Environment | Purpose | Data / Access | Promotion Rule |
| --- | --- | --- | --- |
| Local development | Developer feedback and isolated unit/component work | Synthetic data; local secrets only | Code review and automated checks |
| Integration / Test | Cross-component and automated system tests | Synthetic/approved test images; team access | Versioned build passes integration exit criteria |
| Pilot / Production-like | Field pilot and UAT | Authorized farm data; restricted admin access | G4 approval, backup/rollback and support readiness |

Configuration and credentials are environment-specific; artifacts are immutable and promoted rather than rebuilt differently.

Each release has a semantic version, source commit/tag, database migration set, firmware/model versions, dependency lockfiles, checksums, release notes and rollback plan.

Database migrations are reviewed for backward compatibility, backup requirement, execution time and rollback/forward-fix approach.

# 2. Release Manifest

| Item | Required Value / Evidence |
| --- | --- |
| Release ID and date | LNTB-vX.Y.Z, planned/effective timestamp |
| Source versions | Backend commit/tag, Flutter commit/tag, firmware commit/tag, AI model/dataset version |
| Artifacts | Backend package/container reference, signed Android artifact checksum, firmware binaries/checksums, model checksum |
| Dependencies/config | Lockfiles, runtime versions, migration list, non-secret configuration schema |
| Quality | Test report, open defect list, security review and QA recommendation |
| Operations | Deployment steps, health checks, monitoring, backup and rollback reference |
| Approval | Technical, QA, Operations and Sponsor/Product gate decision as applicable |

# 3. Laravel Deployment Runbook

Confirm approved release manifest, maintenance window, backup freshness and rollback/forward-fix plan.

Verify host/runtime, TLS, database, cache/queue, Reverb, storage and required environment variables without printing secret values.

Enable appropriate maintenance controls if required; deploy immutable code/artifact and install production dependencies from lockfile.

Run reviewed database migrations with logging; stop and execute approved recovery if a critical migration fails.

Refresh optimized configuration/routes/events only according to the release procedure; restart queue workers and Reverb gracefully.

Verify scheduler heartbeat, queue workers, storage access, health endpoints, login, telemetry ingestion, command flow and private realtime authorization.

Monitor error rate, latency, queue failures and device sync; record effective version and completion decision.

# 4. Device Provisioning and Field Installation

| Step | Control / Evidence |
| --- | --- |
| Receipt inspection | Correct board/sensor/meter ratings, no damage, serial/hardware UID recorded |
| Bench assembly | Reviewed wiring, default outputs OFF, fused/protected supply, labelled connectors |
| Flash | Approved firmware binary/checksum, device UID and version recorded |
| Credential provisioning | Unique credential displayed/installed once; server stores protected representation; test then revoke any temporary credential |
| Calibration | Sensor/meter coefficients and date stored against device; acceptance results attached |
| Site installation | Waterproof enclosure, cable glands, isolation, physical cutoff, Wi-Fi test and safe mechanical mounting |
| Pairing | Assign to correct farm; verify telemetry, command, event, realtime and notification |
| Handover | Farmer safe-use briefing, support contact, manual fallback and maintenance schedule |

# 5. Flutter Android Release Runbook

Build from tagged source with pinned Flutter/Dart/package versions and release environment configuration.

Run static analysis, unit/widget/integration tests, localization completeness and accessibility checks.

Generate signed release artifact using protected signing material; never store signing secrets in repository or documentation.

Install on supported reference devices, including the selected low-end Android device; test login, cache, command, Reverb and notifications.

Record version/build number, checksum, supported OS/device notes, release notes and distribution location.

For pilot distribution, maintain authorized recipient list and rollback to a known compatible build when required.

# 6. AI Model Release

| Control | Requirement |
| --- | --- |
| Model card | Purpose, classes, dataset version, collection conditions, metrics, limitations and intended/non-intended use |
| Artifact integrity | Model checksum, framework/runtime version, preprocessing contract and label map |
| Compatibility | Backend inference contract and resource benchmark verified |
| Canary/validation | Run known sample set and compare expected output before activation |
| Rollback | Previous model retained; model version referenced by every result |
| Monitoring | Confidence/class distribution and reviewed disagreements tracked |

# 7. Rollback and Recovery Matrix

| Failure | Immediate Action | Recovery Decision |
| --- | --- | --- |
| Backend application defect | Disable affected feature or redeploy previous compatible artifact | Rollback if schema remains compatible; otherwise approved forward fix |
| Migration failure | Stop deployment, preserve logs and prevent mixed-version writes | Restore backup or forward-correct per reviewed plan |
| Firmware unsafe behavior | Physically isolate/disable actuation; revoke commands if needed | Flash previous approved firmware after bench verification |
| Mobile incompatible release | Stop distribution; provide previous compatible APK/build | Verify API compatibility and token/data migration |
| AI quality regression | Deactivate model/return prior model; queue or manually review results | Restore previous model and investigate dataset/preprocessing |
| Credential exposure | Revoke/rotate affected credential and inspect audit logs | Re-provision, validate and document incident |

# 8. Monitoring and Alerts

| Signal | Threshold / Trigger | Response |
| --- | --- | --- |
| API availability/error rate | Health fail or sustained elevated 5xx | Operations triage; protect commands; incident if core flow affected |
| API latency | p95 above pilot threshold | Check DB, queue, network and recent release |
| Queue failed/depth | Failed job or growing backlog | Retry/diagnose provider/worker; verify idempotency |
| Scheduler heartbeat | Missing beyond expected interval | Restore scheduler; rerun idempotent due jobs |
| Reverb connectivity | Service/channel authorization failures | Restart/inspect config; clients resync by HTTP |
| Device freshness | No telemetry beyond configured window | Notify farmer/support; check power/Wi-Fi/device |
| Offline sync backlog | Backlog not decreasing after reconnect | Inspect credential, sequence, storage and server errors |
| Pump safety event | Max runtime/fault/reboot during event | Immediate farm review and device lockout as required |
| Backup freshness/restore test | Older than RPO or test failed | Escalate; fix before pilot/release |

# 9. Routine Operations and Maintenance

| Frequency | Activity | Evidence |
| --- | --- | --- |
| Daily during pilot | Review health, queue/scheduler, device freshness, critical alerts and backup completion | Operations log |
| Weekly | Inspect synchronization gaps, error trends, storage, costs and open incidents/defects | Weekly status |
| Monthly / before field milestone | Restore test, credential/access review, dependency/security review and capacity check | Control checklist |
| Per growing cycle / as conditions change | Recalibrate sensors/meters; inspect enclosure, wiring, pump and camera position | Calibration/maintenance record |
| Per release | Update runbooks, release manifest, compatibility matrix and known issues | Release archive |

# 10. Support Model

| Support Level | Scope | Target Response | Escalation |
| --- | --- | --- | --- |
| L1 — Farmer / Project Support | Usage questions, connectivity check, app restart, safe manual fallback | Business hours / pilot contact agreement | To L2 with device/farm/time/screenshot |
| L2 — Technical Team | Account/device config, logs, synchronization, deployment and hardware inspection | Critical immediate; high same working day | To domain lead / incident process |
| L3 — Engineering | Code, firmware, AI model, database or security defect | Per severity and release decision | Sponsor for major risk/scope/cost |

# 11. Decommissioning

Safely disconnect pump/actuators and power; remove or transfer hardware with inventory record.

Revoke user/device/provider credentials and signing/distribution access no longer required.

Export agreed records, apply deletion/retention decisions and verify private image lifecycle.

Archive source, releases, model/calibration evidence, approvals and operations history.

Communicate service end date, support owner and any manual farming procedure to users.

# Authoritative References

| ID | Source | Use |
| --- | --- | --- |
| SRC-01 | LNTB Khmer proposal | Goals, scope, BOM and timeline |
| SRC-02 | LNTB requirements | Functional/non-functional baseline |
| SRC-03 | LNTB project structure | Initial architecture, data and APIs |
| SRC-04 | Official Laravel docs | Backend implementation baseline |
| SRC-05 | Official Flutter docs | Mobile architecture/quality baseline |
| SRC-06 | Official Espressif docs | ESP32 hardware/network constraints |