LNTB

Smart Tomato Farming System

Requirements Specification Document

Team: LNTB (Leading Next-Gen Tomato Business)

Stack: Laravel · Flutter · Arduino (ESP32)

# 1. Introduction

## 1.1 Purpose

This document defines what each part of the LNTB system must do and the conditions it must operate under. It covers functional requirements, non-functional requirements, hardware requirements, software requirements, and data requirements across all three components: IoT firmware, Laravel backend, and Flutter mobile app.

## 1.2 Scope

The system helps Cambodian net-house tomato farmers automate irrigation, track water and electricity cost in real time, and detect fruit ripeness using a sensor and AI camera setup, all managed through a Khmer-language mobile app.

## 1.3 Definitions

| Term | Meaning |
| --- | --- |
| Net house | Netted greenhouse structure for tomato cultivation |
| Sensor node | ESP32 board + moisture/temp/light sensors + relay/pump |
| Camera node | ESP32-CAM for ripeness detection |
| Farmer | Primary end user of the mobile app |

# 2. Overall System Requirements

- The system shall consist of three cooperating components: firmware (Arduino/ESP32), backend (Laravel), and mobile app (Flutter).

- The system shall function with intermittent or weak WiFi typical of rural net-house locations.

- The system shall operate on a total hardware budget of approximately $103.50 per net house.

- The system shall support at least one net house per farmer account at MVP stage.

# 3. Functional Requirements

## 3.1 Firmware (Arduino / ESP32) — FR-HW

| ID | Requirement |
| --- | --- |
| FR-HW-01 | The sensor node shall read soil moisture at a configurable interval (default: every 60s). |
| FR-HW-02 | The sensor node shall read temperature and humidity via DHT22. |
| FR-HW-03 | The sensor node shall read ambient light level via LDR. |
| FR-HW-04 | The sensor node shall send readings to the Laravel API as JSON over HTTPS. |
| FR-HW-05 | The sensor node shall poll the Laravel command endpoint (default: every 10s) for pending commands. |
| FR-HW-06 | The sensor node shall trigger the relay (water pump) on pump_on / pump_off commands. |
| FR-HW-07 | The sensor node shall trigger the servo (net roof) on servo_open / servo_close commands. |
| FR-HW-08 | The sensor node shall acknowledge executed commands back to the backend. |
| FR-HW-09 | The sensor node shall fall back to a local safe moisture threshold if backend is unreachable. |
| FR-HW-10 | The camera node shall capture an image of tomato fruit at a configurable interval or on trigger. |
| FR-HW-11 | The camera node shall send the captured image or inferred ripeness result to the Laravel API. |
| FR-HW-12 | The firmware shall store WiFi credentials and API endpoint in a separate config file. |
| FR-HW-13 | The firmware shall reconnect automatically after WiFi drop without manual reset. |

## 3.2 Backend (Laravel) — FR-BE

| ID | Requirement |
| --- | --- |
| FR-BE-01 | The backend shall expose an authenticated endpoint for ESP32 devices to submit sensor readings. |
| FR-BE-02 | The backend shall expose an endpoint for ESP32 to poll pending commands. |
| FR-BE-03 | The backend shall expose an endpoint for ESP32 to acknowledge command execution. |
| FR-BE-04 | The backend shall expose an endpoint to receive ripeness detection results/images. |
| FR-BE-05 | The backend shall calculate real-time water (m³) and electricity (kWh) usage per irrigation event. |
| FR-BE-06 | The backend shall run a scheduled job to roll up daily usage into usage_records with USD cost. |
| FR-BE-07 | The backend shall broadcast live sensor updates via WebSocket (Laravel Reverb) per farm channel. |
| FR-BE-08 | The backend shall send push notifications (FCM) when a fruit is marked ready-to-harvest. |
| FR-BE-09 | The backend shall send push notifications when soil moisture crosses a critical threshold. |
| FR-BE-10 | The backend shall provide Sanctum token-based authentication for the Flutter app. |
| FR-BE-11 | The backend shall provide paginated, filterable logbook query endpoints (month/year/range). |
| FR-BE-12 | The backend shall allow a farmer to manually trigger/stop irrigation via authenticated API. |
| FR-BE-13 | The backend shall record who/what triggered each irrigation event (auto vs manual). |
| FR-BE-14 | The backend shall support configurable per-farm cost rates ($/m³, $/kWh). |

## 3.3 Mobile App (Flutter) — FR-APP

| ID | Requirement |
| --- | --- |
| FR-APP-01 | The app shall display a login screen and authenticate against Laravel Sanctum. |
| FR-APP-02 | The app shall display a real-time dashboard: moisture, temperature, humidity, light. |
| FR-APP-03 | The app shall visually indicate status using color coding (green = normal, red = attention). |
| FR-APP-04 | The app shall allow the farmer to manually trigger or stop irrigation. |
| FR-APP-05 | The app shall display current auto-irrigation status (on/off, last triggered). |
| FR-APP-06 | The app shall display the latest ripeness detection results with images and confidence. |
| FR-APP-07 | The app shall display real-time and historical water/electricity cost. |
| FR-APP-08 | The app shall provide a digital logbook, filterable by month and year. |
| FR-APP-09 | The app shall receive and display push notifications for ripeness and threshold alerts. |
| FR-APP-10 | The app UI shall be rendered 100% in Khmer language. |
| FR-APP-11 | The app shall function acceptably on low-end/budget Android devices. |
| FR-APP-12 | The app shall handle intermittent connectivity gracefully (cache + retry). |

# 4. Non-Functional Requirements

| ID | Category | Requirement |
| --- | --- | --- |
| NFR-01 | Usability | A farmer with no technical background can read dashboard status within 5 seconds via color/icon cues. |
| NFR-02 | Performance | Sensor readings appear on the dashboard within 5 seconds of being sent (via WebSocket). |
| NFR-03 | Reliability | Basic auto-irrigation continues even if the Laravel backend is temporarily unreachable. |
| NFR-04 | Availability | Backend targets uptime suitable for a single-farm pilot (no formal SLA at MVP). |
| NFR-05 | Security | All API traffic uses HTTPS. Device endpoints use API keys; user endpoints use Sanctum. |
| NFR-06 | Scalability | Schema supports multiple farms/devices per user without redesign. |
| NFR-07 | Localization | All user-facing text is sourced from Khmer .arb files, never hardcoded. |
| NFR-08 | Cost | Total hardware BOM per net house does not exceed approximately $105 USD. |
| NFR-09 | Maintainability | Firmware, backend, and app are structured into clearly separated modules. |
| NFR-10 | Data retention | Sensor readings and logs are retrievable for at least one full growing season. |

# 5. Hardware Requirements

| Component | Purpose | Qty |
| --- | --- | --- |
| ESP32 Microcontroller Board | Main sensor node controller | 1 |
| ESP32-CAM | Ripeness detection camera node | 1 |
| Soil Moisture Sensor | Soil moisture reading | 2 |
| DHT22 | Temperature & humidity | 1 |
| LDR Sensor + Servo Motor | Light reading + auto net roof control | 1 set |
| 2-Channel Relay + Water Pump + Fan | Irrigation + ventilation control | 1 set |
| Wiring / waterproof enclosure / circuit board | Assembly | — |

Environmental requirement: all outdoor / net-house hardware (sensor node, wiring, camera) must be housed in a waterproof enclosure suitable for humid greenhouse conditions.

# 6. Software Requirements

| Layer | Item | Choice |
| --- | --- | --- |
| Firmware | Language | C++ (Arduino Framework) |
| Firmware | Toolchain | PlatformIO (recommended) or Arduino IDE |
| Firmware | Key libraries | DHT sensor library, ESP32Servo, HTTPClient, ArduinoJson, esp32-camera |
| Backend | Framework | Laravel (latest LTS-compatible) |
| Backend | Auth | Laravel Sanctum |
| Backend | Realtime | Laravel Reverb |
| Backend | Database | MySQL |
| Backend | Notifications | Firebase Cloud Messaging |
| Mobile App | Framework | Flutter |
| Mobile App | State management | Riverpod |
| Mobile App | Routing | GoRouter |
| Mobile App | Realtime client | laravel_echo + pusher_client |
| Mobile App | HTTP client | Dio |
| Mobile App | Localization | Flutter intl / .arb files (Khmer) |

# 7. User Requirements

- Farmers using the app may have limited literacy with technology — UI must rely on color/icons over dense text.

- Farmers need to check farm status remotely (not physically present at net house at all times).

- Farmers need historical cost data to compare month-to-month or season-to-season.

- Farmers need confidence that manual override is always possible, even when auto-mode is active.

# 8. Data Requirements

- Sensor readings must be timestamped and traceable to a specific device and farm.

- Irrigation events must record enough detail (duration, volume, trigger source) to support accurate cost calculation.

- Ripeness results must retain the source image for farmer verification (AI is a decision aid, not fully autonomous).

- Daily usage rollups must persist independently of raw readings so historical cost comparisons remain fast.

# 9. Interface Requirements

- Firmware ↔ Backend: HTTPS/JSON over REST.

- Backend ↔ Mobile App: HTTPS/JSON REST for requests, WebSocket (Reverb) for live push, FCM for notifications.

- No direct communication between firmware and Flutter app — Laravel is the sole intermediary, by design.

# 10. Constraints & Assumptions

## Constraints

- Budget ceiling (~$105/net house) restricts sensor precision and camera quality.

- Rural WiFi reliability cannot be assumed to be constant.

- Team is early-stage in Laravel/Flutter/Arduino integration experience — architecture favors simplicity.

## Assumptions

- Each net house has at least intermittent WiFi coverage.

- Farmers have access to a smartphone capable of running a modern Flutter app.

- One ESP32-CAM per net house is sufficient for ripeness sampling (not full 100% fruit coverage).

# 11. Open Items Requiring Decision

- AI ripeness inference location: on ESP32-CAM vs. Laravel-side/external inference service.

- Whether moisture threshold values are firmware-hardcoded-only or backend-configurable.

- Confirm single-farm vs multi-farm MVP scope.